## Purpose:

The purpose of this PowerShell script is to audit multiple workstations on a network to determine whether a specific Windows service (such as Windows Update, antivirus, etc.) is running properly. This is useful for system administrators to:

- Ensure critical services are running across the organization.

- Detect stopped or failed services that may affect system stability or security.

- Generate a report for compliance or troubleshooting.

Function :

The script performs the following tasks in order:

- Defines the service name to be checked (e.g., "wuauserv" for Windows Update).

- Loads a list of workstation names, either hardcoded or from a file.

- Iterates through each workstation, remotely checking the status of the specified service.

- Handles errors gracefully if a workstation is unreachable or the service doesn't exist.

- Stores results (computer name, service name, and status) in a table.

- Displays the results in a formatted table on the screen.

Usuage:

- Open PowerShell as Administrator.

- Modify the $ServiceName and $Workstations list in the script (or load from a file).

- Run the script:

Script:

# File: Audit-ServiceStatus.ps1

# Define the service name you want to audit

$ServiceName = "wuauserv"

# List of workstations

$Workstations = @(

"WS001",

"WS002",

"WS003"

)

# Create an array to store results

$Results = @()

foreach ($Computer in $Workstations) {

Write-Host "Checking $ServiceName on $Computer..." -ForegroundColor Cyan

try {

$Service = Get-Service -ComputerName $Computer -Name $ServiceName -ErrorAction Stop

$Results += [PSCustomObject]@{

ComputerName = $Computer

ServiceName  = $ServiceName

Status       = $Service.Status

}

}

catch {

$Results += [PSCustomObject]@{

ComputerName = $Computer

ServiceName  = $ServiceName

Status       = "Error: $_"

}

}

}

# Output the results to the console

$Results | Format-Table -AutoSize